## 4.4.1 Timing for pixel columns

Fig. 4.4.1 The readout timing sequence of one double column.

| Timing definition in Fig. 4.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Symbol | Description | Value (ns) |  |  |
|  |  | min | typ | max |
| Tclk | Cycle of system clk |  | 23 |  |
| TWA | Wait time for different TOAs, larger than time walk | 300/200? |  |  |
| TCLR | The clearance between different pixels | 0 |  |  |
| Tdin | The input delay required by the peripheral circuits | - | - | 6 |
| Tsu | Setup time required by the latch at the end of double column |  |  | 1 |
| Thd | Hold time required by the latch at the end of double column | 1 |  |  |
| TDFOR | The delay time of Fastor referring positive edge of fastor. 0<TDAP<12Tclk-Tsu | 0 | - | 172 |
| TDAP | The address delay at the positive edge of READ 0<TDAP<4Tclk-Tsu |  |  | 80 |
| TDAN | The address hold time at the negative edge of READ ( in order to satisfy the hold time of the addr latch). TDAN>Thd | 1 |  |  |
| Signal description in Fig. 4.4.1 |  |  |  |  |
| Name | Description | Note |  |  |
| Fastor（EOC） | OR results of all the pixel digital output in a double column. EOC means end of column, the nearest position to the peripheral circuits. | Interface signal from pixel array |  |  |
| FRERZE(EOC) | Freeze the pixel state, avoid the new hit disturbing the present readout. | Interface signal from periphery |  |  |
| READ（EOC） | Hit address read signal. Reset the hit pixel when high. READ(EOC) indicates the read signal just sent from the peripheral circuits. | Interface signal from periphery |  |  |
| READ（top） | Hit address read signal. Reset the hit pixel when high. READ(top) indicates the read signal arrived the farthest pixel. | Signals in Pixel array |  |  |
| ADDR_FEI3_i | The address before addr latch | Signals in Pixel array |  |  |
| PullN | The pull up signal for the ADDR_FEI3_i, low active. 低有效，控制将地址拉高。 | Interface signal from periphery |  |  |
| Latch_en | The latch en of the addr latch, high active. 高数据通过，低数据保持。 | Interface signal from periphery |  |  |
| ADDR_FEI3_o | The address after addr latch, which will be sent to the peripheral circuits | Interface signal from pixel array |  |  |
| CLK | The main clock, 40 MHz in FCC, 43.33MHz in CEPC | Signals in Periphery |  |  |
| CLK20M | Generated clock, frequency is half of CLK | Signals in Periphery |  |  |
| Fastor_syn | The synchronized Fastor in the peripheral signal | Signals in Periphery |  |  |
| Latch_en_Per | Latch enable in the peripheral circuits. The data will be taken at the positive edge of CLK when Latch_en_Per is hign. | Signals in Periphery |  |  |
| Addr_rcv | The received addr in the peripheral circuits | Signals in Periphery |  |  |